Custom ESP-IDF Installation Tutorial

Overview

This tutorial provides a step-by-step guide for installing ESP-IDF with full control over the installation process. This approach enables you to:

- Run multiple ESP-IDF versions independently for ESP32 development

- Create custom shortcuts for common commands (e.g., flash4 instead of idf.py -p COM4 flash monitor)

- Use PowerShell Core in VS Code terminal for all development tasks

- Have complete control over installation directories for IDF versions and toolchains

Prerequisites

- Windows 10/11

- Git installed and accessible from command line

- Administrative privileges for PowerShell execution policy changes

- Visual Studio Code (recommended)

Step 1: Prepare Directory Structure

- Create two main folders:

  - D:\IOT\IDF55 (for ESP-IDF source code)

  - D:\IOT\IDF55Tools (for toolchain)

Note: Adjust paths according to your preferred location

Step 2: Download ESP-IDF Source Code

- Open Windows PowerShell

- Navigate to the IDF folder: cd D:\IOT\IDF55

- Clone the specific ESP-IDF version: git clone -b v5.5 --recursive https://github.com/espressif/esp-idf.git

- Important: Move all contents from the created esp-idf-v5.5 subfolder directly into D:\IOT\IDF55 to ensure scripts work correctly.

Step 3: Configure PowerShell Execution Policy

Set the execution policy to allow script execution:

Set-ExecutionPolicy -ExecutionPolicy Bypass -Scope CurrentUser

Step 4: Install ESP-IDF Toolchain

- Set the custom tools path (temporary for current session):

- $Env:IDF_TOOLS_PATH = "D:\IOT\IDF55Tools"

- Verify the path is set correctly:

- $Env:IDF_TOOLS_PATH

- Install the toolchain from within the IDF directory:

- cd D:\IOT\IDF55

- .\install.ps1

- Set up environment variables:

- .\export.ps1

Note: Use .\export.ps1 (with dot-slash) to execute the script in the current directory

Step 5: Install and Configure PowerShell Core

Install PowerShell Core

- Download PowerShell Core from the official Microsoft repository

- Install the latest version (PowerShell 7.x recommended)

Configure VS Code to Use PowerShell Core

- Open VS Code settings (File → Preferences → Settings)

- Click "Open Settings (JSON)" in the top-right corner

- Add or modify the terminal configuration: {    "terminal.integrated.defaultProfile.windows": "PowerShell",    "terminal.integrated.profiles.windows": {        "PowerShell": {            "source": "PowerShell",            "path": "C:\\Program Files\\PowerShell\\7\\pwsh.exe"        }    }}

Step 6: Create PowerShell Profile with Custom Functions

- Locate your PowerShell profile:

- $PROFILE

- Create or edit the profile file (use Notepad++ or VS Code):

- notepad $PROFILE

- Add the ESP-IDF initialization function:

- function idf55 {

- $Env:IDF_TOOLS_PATH = "D:\IOT\IDF55Tools"

- . D:\IOT\IDF55\export.ps1

- }

- Add useful shortcut functions:

- # Monitor functions

- function monit3 { idf.py -p COM3 monitor }

- function monit4 { idf.py -p COM4 monitor }

- function monit5 { idf.py -p COM5 monitor }

- function monit6 { idf.py -p COM6 monitor }

- # Flash and monitor functions

- function flash3 { idf.py -p COM3 flash monitor }

- function flash4 { idf.py -p COM4 flash monitor }

- function flash5 { idf.py -p COM5 flash monitor }

- function flash6 { idf.py -p COM6 flash monitor }

- # Erase flash functions

- function ef3 { idf.py -p COM3 erase-flash }

- function ef4 { idf.py -p COM4 erase-flash }

- function ef5 { idf.py -p COM5 erase-flash }

- function ef6 { idf.py -p COM6 erase-flash }

- # Utility functions

- function fclean { idf.py fullclean }

- function getsize { idf.py size-components }

- # Port listing function

- function prts {

- $portList = Get-PnpDevice -Class Ports -ErrorAction SilentlyContinue

- if ($portList) {

- foreach($device in $portList) {

- if ($device.Present) {

- Write-Host $device.Name "(Manufacturer: $($device.Manufacturer))"

- }

- }

- }

- }

- # Create new project function

- function cproj {

- $ProjectName = Read-Host -Prompt 'Enter new project name'

- idf.py create-project $ProjectName

- }

- # Target selection functions

- function c3 { idf.py set-target esp32c3 }

- function s3 { idf.py set-target esp32s3 }

- function esp32 { idf.py set-target esp32 }

Step 7: Configure VS Code for ESP-IDF Development

Update IntelliSense Configuration

- In your project folder, create or update .vscode/c_cpp_properties.json: {    "configurations": [        {            "name": "ESP-IDF",            "includePath": [                "${workspaceFolder}/**",                "D:/IOT/IDF55/components/**"            ],            "defines": [                "_DEBUG",                "UNICODE",                "_UNICODE"            ],            "cStandard": "c17",            "cppStandard": "gnu++17",            "intelliSenseMode": "gcc-x64"        }    ],    "version": 4}

Configure IntelliSense Engine

- Open VS Code settings

- Search for "C_Cpp.intelliSenseEngine"

- Set it to "Tag Parser" for better performance with ESP-IDF projects

Step 8: Usage Instructions

- Start a new development session:

  - Open VS Code

  - Open terminal (PowerShell Core should be default)

  - Run: idf521

  - You should see ESP-IDF environment activation messages

- Use custom shortcuts: Once your PowerShell profile is loaded, you can use simple commands instead of typing full ESP-IDF commands:

  - Type flash3 and press Enter → Executes idf.py -p COM3 flash monitor

  - Type flash4 and press Enter → Executes idf.py -p COM4 flash monitor

  - Type monit4 and press Enter → Executes idf.py -p COM4 monitor

  - Type ef3 and press Enter → Executes idf.py -p COM3 erase-flash

  - Type prts and press Enter → Lists available COM ports

  - Type cproj and press Enter → Create a new project interactively

  - Type fclean and press Enter → Executes idf.py fullclean

  - Type c3 and press Enter → Executes idf.py set-target esp32c3

- Example workflow:

- powershell

- # Initialize ESP-IDF environment

- idf521

- # Check available ports

- prts

- # Flash your project to COM4 and start monitoring

- flash4

Troubleshooting

Common Issues and Solutions

Problem: export.ps1 not recognized

- Solution: Ensure you're in the correct directory and use .\export.ps1

Problem: Tools not found after running idf55

- Solution: Verify IDF_TOOLS_PATH is correctly set and toolchain installation completed successfully

Problem: IntelliSense not working

- Solution: Check include paths in c_cpp_properties.json and ensure Tag Parser is selected

Problem: PowerShell execution policy errors

- Solution: Run Set-ExecutionPolicy -ExecutionPolicy Bypass -Scope CurrentUser

Benefits of This Setup

- Multiple versions: Easy switching between different ESP-IDF versions

- Portable: Self-contained installation with known paths

- Efficient workflow: Custom shortcuts reduce typing and increase productivity

- Professional environment: PowerShell Core provides better scripting capabilities

- Full control: Complete visibility and control over the installation process

Adding Additional ESP-IDF Versions

To add another version (e.g., v5.3.0):

- Create new folders: IDF530 and IDF530Tools

- Repeat the installation process with the new version

- Add a new function to your PowerShell profile: function idf530 {    $Env:IDF_TOOLS_PATH = "D:\IOT\IDF530Tools"    . D:\IOT\IDF530\export.ps1}

This approach allows you to maintain multiple ESP-IDF versions simultaneously and switch between them as needed for different projects.